## High-Level Architecture of the rudder-pi Ship Software System

### 1) Android “rudderpi” App (runs on the Android smartphone inside the ship)

The Android “rudderpi” App is responsible for telemetry acquisition, video capture, and network connectivity.

It consists of the following components:

- RTSPLibrary
A lightweight RTSP streaming library providing RTSP client functionality.
The smartphone camera stream is sent via RTSP to the Raspberry Pi.

- NanoHTTPD
A minimal embedded HTTP server used to expose telemetry data (e.g. GPS, compass, sensor values) via a state.json endpoint.

- Mobile Network Connectivity
A 4G/LTE service provider and SIM card inside the smartphone provide internet access for the entire ship system.

- USB-C Ethernet Adapter
An external USB-C LAN adapter connects the Android smartphone via wired Ethernet directly to the Raspberry Pi inside the ship.

The Android smartphone therefore acts as:

- a sensor hub

- a video source

- and a network gateway for the ship

### 2) Python “rudder-pi” Service (runs on the Raspberry Pi inside the ship)

The Python “rudder-pi” Service is the central control and integration layer of the ship.

It consists of the following components:

- MediaMTX Server
A media relay and protocol conversion server.
It receives the RTSP video stream from the Android smartphone and converts it into a WebRTC stream for low-latency browser-based viewing.

- Gradio (Python package)
Provides the web-based ship control interface used by the remote operator.
This includes steering controls, status display, and video embedding.

- Folium (Python package)
Used to display a map with the current GPS position of the ship inside the remote control web interface.

- GPIO Control Layer
A Python GPIO library used to control motors, rudder servos, and other actuators connected to the Raspberry Pi.

- Python HTTP Client
Periodically fetches telemetry data from the Android “rudderpi” App by requesting the state.json endpoint.

The Raspberry Pi therefore acts as:

- the control computer

- the video distribution node

- and the interface between sensors and actuators

### 3) WireGuard VPN Server

An external WireGuard VPN Server securely connects all system components.

It provides:

- encrypted communication between the Raspberry Pi

- the remote control device

- and, indirectly, the Android smartphone via the ship network

This ensures:

- secure remote access

- stable addressing

- independence from mobile network NAT limitations

### 4) Remote Control Device (operator side)

An arbitrary Remote Control Device (laptop, tablet, or desktop computer) used by the operator.

- Connected to the WireGuard VPN

- Accesses the web-based control interface hosted by the Raspberry Pi

- Displays:

  - live WebRTC video

  - ship telemetry

  - map position

  - and control inputs

## Object Naming Summary (Consistency Reference)

- Android “rudderpi” App → always refers to the Android application

- Android smartphone → the physical device inside the ship

- Python “rudder-pi” Service → the Python software running on the Raspberry Pi

- Raspberry Pi → the onboard control computer

- MediaMTX Server → RTSP → WebRTC video conversion

- WireGuard VPN Server → secure network backbone

- Remote Control Device → operator’s computer or tablet